Lab:03 Integration of Photo Voltaic (PV) System in Electrical Grid

Objectives:

- To understand the design parameters of a PV system.

- To simulate a simple power grid under normal operating conditions.

- To integrate a PV system with the power grid and analyze its impact.

Introduction:

This lab is designed to develop the basic understanding of important parameters for the design and implementation of a PV system using ETAP.

Step-by-Step Implementation:

Step 1: Create a New Project

- Open ETAP and create a new project:

- Go to File > New Project.

- Name the project "PV-Integration".

- Set the base voltage and frequency for your system (60 Hz).

Step 2: Configure the Power Grid/ Generating Station

- Drag and drop a power grid from the AC components toolbar.

- Double-click on the grid and set its configuration:

  - Swing in the Info tab.

  - Change the ID to Power Grid in the Info tab.

  - Set the Rated KV to 3 in the rating tab.

  - Set MVAsc to 1000 in the 3-Phase section of Short Circuit tab.

  - Set X/R to 40 in the Short Circuit tab.

Step 3: Configure Receiving Station Step-Down Transformer

- Drag and drop a 2-winding transformer from the AC components tool bar.

- Double-click on the transformer and set its configuration:

  - Change the ID to T-1 in the Info tab.

  - Ensure that the standard is set to ANSI.

  - Set the Voltage Rating to 33 KV for Primary and 6.6 KV for Secondary in the Voltage Rating section of the Rating tab.

  - Set the Rated MVA to 1 in the Power Rating section of the Rating tab.

  - Set the Impedance to Typical in the Impedance section of the Impedance tab.

  - Set the Grounding to Y configuration in the Grounding tab.

Step 4: Configure Cable-1

- Drag and drop a cable from the AC components tool bar.

- Double-click on the cable and set its configuration:

  - Change the ID to Cable-1 in the Info tab.

  - Set the length of cable to 1 km.

  - Go to the Cable Library in the Info Tab and select a 6.6 KV cable, with CU and 3/C type and a size of 400 mm2 respectively.

Step 5: Configure Cable-2

- Drag and drop a cable from the AC components tool bar.

- Double-click on the cable and set its configuration:

  - Change the ID to Cable-2 in the Info tab.

  - Set the length of cable to 1 km.

  - Go to the Cable Library in the Info Tab and select a 6.6 KV cable, with CU and 3/C type and a Phase of 300 mm2 respectively.

Step 6: Configure the Load

- Drag and drop a Static Load from the AC components tool bar.

- Double-click on the load and set its configuration:

  - Change the ID to Load-1 in the Info tab.

  - In the Loading tab select unit to KV to 6.6, KVA to 4 and kW to 3.58 respectively.

- Drag and drop a Lumped Load from the AC Components tool bar.

- Double-click and set its configuration:

  - In the Nameplate tab select unit to kVA to 2.5, kW to 2.13 respectively.

Step 7: Add Busses

- Drage and drop bus from the tool bar and insert it in between the corresponding components of the power grid.

- Name the busses accordingly and check their voltage levels.

Step 8: Load Flow Analysis

- Change the current mode from Edit to the Load Flow Analysis Mode.

- Click on the Run Load Flow Calculations button.

- Analyze the results:

  - Check voltages at all buses.

  - Check the Voltage at the Transformer.

  - Check for overloading and underloading conditions.

Step 9: Introduction of PV System

- Drag and drop a PV Array Editor from the AC components toolbar at the BUS in between the Cable-1 and Cable-2.

- Double-click on the Editor and set its configuration:

  - Select a suitable PV Panel from the Library section in the PV Panel Tab.

    - For this lab we will select Q Cells from the Manufacturer with a model make of 215-230.

  - In the PV array tab increase the number of PV Panels in Series to 17 and the number of PV Panels in parallel to 2.

  - Click on the Irradiance Calculation button and select a suitable location based on the latitude and longitude parameters.

  - Now click on the Inverter Editor Button in the Inverter Tab and set the DC Rating to 8 kW and V to 6.6 in the Rating tab

Step 10: Load Flow Analysis

- Change the current mode from Edit to the Load Flow Analysis Mode.

- Click on the Run Load Flow Calculations button.

- Analyze the results:

  - Check the improvements.

  - Observe the power flow from the power grid and the PV system.

Lab Report:

- Draft the procedure in your own words in your lab report.

- Write the real power and apparent power appearing at Bus 3 with and without the insertion of PV system.

- Increase the lumped load to 5 KVA and observe the PQ values at BUS 3 before and after the insertion of PV system.

Additional Tasks

- Manually insert the PV type as per the available models today and observe the results.